Antony Sutanto

Suzhou Bay Cultural Centre

Suzhou, Jiangsu, China

Designed by Pritzker-winning French architect Christian de Portzamparc, the Suzhou Bay Cultural Centre is comprised of the Suzhou Opera House and the Wujiang Exhibition Centre united by a 500-meter-long, 40-meter-high metallic ribbon [1]. The ribbon generates a set of two wings that unite the music and performance halls on one side with the museums and pedagogical venues on the other. Their movements consist of twisting, turning, and winding both up and down as they reach from one roof of a building to the other. At the center, the wings are connected to form an opening to the sky in the form of an arch that can be experienced from the top ribbon, which also serves as a pedestrian bridge. A terrace at the center acts as a stage of entry from the water’s edge along which the building is situated while enhancing the visitor’s experience by sheltering a shopping center located along the walk below the ribbon.

The entire center occupies a site of 215,000 m2 and the two buildings that the ribbons house are roughly 40,000 m2 each. The Suzhou Opera House includes 1,600 seats while the Wujiang Exhibition Centre includes a 28,000 m2 museum, a 15,000 m2 exhibition center, and a 20,000 m2 convention center [2]. The ribbons utilize space truss structures with a rotating geometry and cantilevering 50m outwards as its wings. The top ribbon spans 100m and the bottom ribbon stretches over 370m are both the largest in the country. To meet the requirements held in a seismic zone like Suzhou, computational and numerical analyses were performed to identify load paths and ensure structural integrity for the long-spanning structure [3].

To support the ribbons, V-columns, all over 4m in diameter, inserted into a circular concrete column and driven into the ground – the first time this system was used in China. On the interior, inclined reinforced concrete columns and beams were used to support the museum, skylights, and connection bridges while maintaining a hidden structure. In the opera theater, the audience stands were used to help cantilever out the levels of seating, allowing the structure to facilitate the headroom and comfort levels of the large seating requirements. The façade of the opera house includes hanging bamboo columns divided into different areas according to their loadings, and the curtain wall panels were designed in Rhino and Grasshopper to optimize the division of the panels that could improve constructability and reduce costs while retaining the original concept of the visual aesthetic [3].

Consultants involved in the design of the project included Xu Acoustique for acoustics, Arup China for the structure and the façade, Artill for lighting, and Regis Guidnard for landscaping, with And Studio performing construction site supervision [4]. The center’s project was valued at $422 million, the second-highest budget for cultural infrastructure projects completed in 2020 [5].

Footnotes:

[1] “Cultural Centre of Suzhou,” Christian de Portzamparc, December 2020, https://www.christiandeportzamparc.com/en/projects/centre-culturel-suzhou-2/.

[2] van Es, Karl, “Christian de Portzamparc designs water-front cultural center in Suzhou,” Åvontuura, 21 January 2021, https://www.avontuura.com/suzhou-bay-cultural-center-by-christian-de-portzamparc/.

[3] “Designing sweeping metal ribbons to house the Suzhou Cultural Centre,” Arup, https://www.arup.com/projects/suzhou-cultural-centre.

[4] “Suzhou Bay Grand Theater / Christian de Portzamparc,” ArchDaily, 16 December 2020, https://www.archdaily.com/953291/suzhou-bay-grand-theater-christian-de-portzamparc.

[5] “2020 Cultural Infrastructure Index,” AEA Consulting, 3-16, https://aeaconsulting.com/uploads/1300013/1626121163743/AEA_-_Cultural_Infrastructure_Index_2020_-_Web.pdf.