Antony Sutanto

T Park

Hong Kong, Greater China

T Park Hong Kong is a sludge treatment facility designed by Vasconi Architectes as a part of a series of projects in the department of New Territories in Hong Kong, far away from the high-density city neighborhoods. The land is situated between green mountains and the bay of the South China Sea which separates Hong Kong from Shenzhen. This setting influenced the form of the facility by seeking integration and harmony with the surrounding landscape, seen in a green wave lining the entire roof of the building [1].

The 45,000m2 building is made up of four incinerators in two main plant halls, a ten-story administration tower, and an environmental center with recreation pools and community leisure facilities that include a habitat for water birds and a viewing platform. It is the largest facility of its type in the world; sustainable solutions make the facility self-sufficient and allows it to dispose of up to 2,000 tons of sewage sludge per day. After incineration, the sludge is converted into ash and residues, reducing its volume by up to 90%, substantially lowering the quantity of waste disposed and the emission of greenhouse gases by over 200,000 tons each year [2]. The heat generated from these processes is recovered for power generation and all power surpluses are exported to the electricity grid. At full capacity, an excess of 2MW of power is produced daily [3].

Known for its sustainable design, the building attained both LEED Gold certification and BEAM Plus Provisional Platinum rating. In addition to its self-sufficiency, 70% of the site is made up of landscaping or has green roof coverage. There is zero discharge of water from the facility: the water recovered on the site through desalination is used on site for drinking and industrial processes, green roofs utilize rainwater collection, and all wastewater is treated on-site for reuse. Eco-friendly materials were also used in construction to lower the building’s embodied carbon and the project costs, including low emissivity glass for high thermal efficiency performance, recycled plastics in timber and concrete, and pulverized fuel ash (PFA) materials for reusage in landscaping and backfilling [4].

Arup provided energy solutions, water engineering, building information modeling (BIM), and civil engineering services for the facility. When designing the structure of the building, a modular design was employed to improve the constructability of the flowing roof, which is 400m long and spans over the two incinerator plants as well as the administrative tower in the center. To accomplish the complexity of the project, the BIM process was used to facilitate design and construction, including managing delivery routes for equipment, the pipe layout, and the placement of the plant’s various structures on the site [5].

Footnotes:

[1] “T Park Hong Kong,” Architect Magazine: Architectural Design, Zonda Media, 17 March 2019, https://www.architectmagazine.com/project-gallery/t-park-hong-kong.

[2] “Story,” T · PARK, https://www.tpark.hk/en/story/.

[3] “T-PARK,” Vasconi Architectes, 2015, http://www.vasconi.fr/en/projects/project?project_id=33.

[4] “Design,” T · PARK, https://www.tpark.hk/en/design/.

[5] “Power, spa and sludge,” Arup, https://www.arup.com/projects/t-park.