Australia 108, Melbourne, Australia

Essay by Lynne Hamdan

Australia 108 is $500m constructed residential skyscraper designed by Fender Katsalidis Architects in the Southbank precinct of Melbourne, Australia. It was designed in 2015 and completed in 2020 and it has become the tallest building in Australia by the roof height and the second tallest building by full height. It consists of 1,105 apartments over 100 floors and has a total net area of the building is 2,828m2 constructed on a land area of 2,642m2. The plan in 2009 was to replicate the Eureka Tower which was done by Fender Katsalidis; they have used this as “an opportunity to allow the towers to have a dialogue with each other, being partners in the skyline, while each expressing their own identities”.1

Around two thirds up of the building, stands out this major and an eye catcher golden starburst. This feature is cantilevered six meters into the sky and was “inspired by the Commonwealth Star on the Australian flag”.1 This star has become quite the amenities for this residential skyscraper with infinity swimming pools and a sky garden. Some of the elements and materials that are integrated in Australia 108, are glass panels oscillating while they are lined up by white wireframes. The white bands on the wireframes have been programmed to create patterns that makes the Starburst look like as it was glowing in the sky.2

The tower being over 100 floors tall requires having a strong and solid core. With that being said, a concrete core with “6 m tall, 900mm thick outrigger walls located levels 42 to 43 and 68 to 69” is needed to create a structurally sound construct that will be able to transfer extreme lateral loads to the mega- columns and frame. These mega-columns and mega frame are providing support to flat floors that are “two-way post-tensioned plates”. Melbourne is a windy city with winds that almost reach 100km/h. Engineers came together and “did a complete collaborative wind, structural and geotechnical study” to make sure any movement from the ground and the soil interaction will not “exacerbate the overall movement of [Australia 108]”.  Very thick walls that measured up 1,950mm in width were constructed to help strengthen the overall structure’s lack of movement. The structural engineers decided to “liquid column damping” on the 97th level to improve lateral solidity. 3

Merging modernity with nature, canary palms and trees were inputted for a green element that blanketed the parking garage of 10 floors. Australia 108 achieved “35% reduction in greenhouse gas emissions” and demonstrated to achieve “Best Practice in Sustainable Design”.4 Efficient air conditioning, fixtures, fittings to lower the amount of water consumed, as well as usage of selected materials with “low environmental impact”, were all incorporated into the design and construction of the building.  Low carbon electricity was generated onsite while wasted heat was used for the swimming pool and domestic hot water 4; many sufficient ideas were used to help the carbon footprint. Australia 108 is an astonishing building with a striking feature that holds significance in Melbourne and around the world.

1 https://www.archdaily.com/963757/australia-108-fender-katsalidis-architects#:~:text=Text%20description%20provided%20by%20the,a%20height%20of%20319%20metres.

2 https://archello.com/project/australia-108

3https://createdigital.org.au/engineering-australia-108-southern-hemispheres-tallest-residential-building/#:~:text=Australia%20108%20features%20a%20complex,43%20and%2068%20to%2069

4 https://ndy.com/experience/australia-108-southbank-victoria