BMCE Headquarters - Rabat, Morocco
Essay by Basmah Sawalhi

The Banque Marocaine du Commerce Exterieur (BMCE) Headquarters in Rabat, Morocco, and its sister building in Casablanca, are the first Foster + Partners buildings to be finished in Africa and aimed to create an entirely new office experience for its customers and employees. Standing as a new symbol for its company, the Rabat branch stands at two stories tall, both dedicated to offices with a basement for parking and an overall gross area of 655 square meters4. This project blends the contemporary with traditional Moroccan and Islamic architecture and engineering, with a heavy emphasis on energy efficiency and environmental consciousness. 
 	A mixture of modern and traditional cooling technology was implemented to shield BMCE Headquarters from the harsh Moroccan weather, while also taking advantage of the abundant sunlight. The façade consists of a concrete frame and large glazed panels, and an iconic, skylit dome sits on top of the roof allowing the building to be illuminated naturally during the day. It is protected by an elegant, stainless-steel screen, made of a special low-iron mixture designed to stay cool; being 200 mm deep, the steel screen functions as shading and security against the large glass panels without severely limiting natural light1. Bays are also enclosed in glazed panels and an energy-efficient, traditional envelope wraps around the interior. A unique, centuries-old “earth-tube” cooling system was installed, working by drawing air into an empty pipe to be naturally cooled by the Earth and released back into the branch1. 
 	Building materials were selected for sustainable characteristics and prevalence in Moroccan culture, especially through using local craftsmanship. Much of the building is composed of locally sourced materials, such as black granite and gray limestone. Zellige, a historically popular tile in Islamic architecture that is produced and installed locally, decorates the exterior of the building2. Tadelak, a plaster commonly used in Moroccan construction makes up a good amount of the interior; it is valued for being waterproof, durable, and curing with a seamless finish, adding to the building’s beauty and longevity3. 
	Since the completion of the Rabat and Casablanca branches in 2011, an additional branch is currently under construction in Fez1. The designs of each building are unique, drawing on the resources and culture of the city of each branch, while still remaining consistent with its values. This “kit-of-parts” approach allows for exceptional buildings to be efficiently and economically constructed in multiple locations without sacrificing aesthetics or sustainability.

Sources for this paper:

1 https://www.archdaily.com/123045/bmce-headquarters-foster-partners 
2 https://en.wikipedia.org/wiki/Zellij
3 https://en.wikipedia.org/wiki/Tadelakt#cite_note-sutter-2
4 http://www.archidatum.com/projects/bmce-branches-foster-plus-partners/