The Canada Earth Tower

Essay by: Mariana Mercado

The Canada Earth Tower is a multiuse High-rise design currently in the approval stage before it can begin construction in Vancouver, BC, Canada. Set to be about 40 stories tall, it will stand as the world’s tallest hybrid timber building (2022, Canada Earth Tower). It was designed by Perkins + Will Architects to align with the larger effort of Vancouver’s Zero Emissions Building Plan (2020, McKnight). When completed, the building will comprise of 3 subgrade parking levels, public retail and office space on the 6 lower floors, and approximately 200 apartment units in the upper levels. The Earth Tower is exceptionally notable not only because of its mostly timber structure, but also its Zero-Carbon design and human-based interior and landscape design.

The structure of the building is designed to heavily rely upon engineered wood, including Glulam, cross-laminated, and dowel-laminated timber, as the main structural elements. It is classified as a “hybrid timber” building because it will also utilize concrete for its foundation as well as elevator cores for lateral support. The structural firm, Structure Craft, has helped design and built numerous timber-mass buildings across Canada and is top of the industry for these types of custom timber architectural builds.

Carbon Neutrality was one of the driving factors that impacted the design of the Canada Earth Tower. The embodied carbon of wood as a building material sequesters significant amounts of carbon for the entire lifetime of the structure and greatly reduces the greenhouse gas emissions of a building. To add to the carbon negativity of the wooden structure, the Structural Engineering firm sources its timber from local, sustainably managed forests and manufactures its own engineered wooden structural elements. In an attempt to exceed the goals set by Vancouver’s Zero Emissions Building Plan, the Earth tower will avoid all fossil fuels, utilize triple-glazed glass, implement heat recovery systems, and be designed to exploit natural lighting and ventilation to the highest extent feasible. In addition to these measures, photovoltaics will generate approximately a quarter of the tower’s energy (2022, Yeung). Through its sustainability efforts, it is aiming to achieve the Passive House Certification and the Zero Carbon Building Pilot Program set by the Canada Green Building Council.

Beyond being a zero-emission building, Perkins + Will also designed the Canada Tower to improve the human appeal of High-rise living. In collaboration with Canadian landscape architect, Cornelia Hahn Oberlander, the Earth Tower will feature vibrant and natural green spaces to improve the quality of life for its occupants. The building will have south-facing gardens that join community groups of 12+ apartments. Timber, being the most intriguing structural aspect of the project also lends to the livability of the multiuse complex, with exposed wooden structure changing the typical material found in Highrise living.

Works Cited

Canada earth tower: The World's tallest hybrid mass timber skyscrapers. StructureCraft Builders. (n.d.). Retrieved January 21, 2022, from https://structurecraft.com/projects/canada-earth-tower

McKnight, J. (2020, July 28). PPerkins+Will Designs "world's Tallest Hybrid Wood Tower" for Vancouver. Dezeen. Retrieved January 21, 2022, from https://www.dezeen.com/2019/05/29/canadas-earth-tower-perkins-will-vancouver-canada/

Yeung, K. (2022, January 13). Canada's Earth Tower. Perkins&Will. Retrieved January 21, 2022, from https://perkinswill.com/project/canadas-earth-tower/