International Convention Center Complex - Sydney, Australia

Essay by Matthew Villagomez

The International Convention Center was designed by Hassell and Populous. The project was a part of the revitalization of Sydney’s Darling Harbor. The ICC Sydney complex mainly features ICC Sydney Theater and Exhibition Center along with the International Convention Center. The ICC Sydney complex used 98% of recycled materials from the demolition of Darling Harbor. The Delta Group used 100,000 tons of concrete with 48,000 tons of that being crushed onsite and 16,000 tons of scrap metal4. The ICC Sydney resides between Tumbalong Boulevard and Darling drive and rests along Cockle Bay. The project value is 1.5$ Billion2.

The convention center lies mostly on the harbor and borders Cockle Bay. The building in total boasts 35,000 square meters2. The forecourt is a Spiral Foundation by Robert Woodward from 19881. The main facade of the building is a stunning glass display. The glass on the facade is gridded and angled to create the middle facade of the building to be the focal point. This parametric glass design allows for glinting light from the bay to be refracted to reveal a beautiful display of light. The cantilever roof is designed in a way to mimic the refracted light from the bay. Inside the convention center is a 2,000-capacity grand ballroom and 2,500 capacity theater2.

The Exhibition Center is unique in that it has two levels of exhibition halls with separate loading bays for each level. With three more exhibition halls, the two below mean that the lower levels have to consist of more structural columns1. These lower-level structural columns are widely spaced and provide organization to the two exhibition halls. The three upper exhibition levels are free of structure and feature one-hundred-meter-wide cambered steel roof trusses1. The trusses having a generous back span allows for a great cantilever extension of the roof towards the east. The floors of the upper halls are 20 kPa capacity floors2. The upper halls are serviced by a ramp which frees up public space in the complex. The exhibition building boasts 40,000 square meters2.

The Theater has a very definitive form. Composed of dramatic glazed and screened surfaces create a unique space at the end of the complex near Tumbalong Park. The theaters feature 8,000 seats by a classic fan-shaped plan and staggered banks of seating2. The main entrance is denoted by the glazed and screened surface peeled off to reveal a glass-enclosed building.

The ICC complex provides a 520kW photovoltaic array3. This solar array produces enough energy to power up to 100 homes. It also produces enough energy for 5% of the baseline energy for ICC Sydney3. The project produces 20% less greenhouse gas emissions compared to the Building Code of Australia minimum requirements, 14% less in total potable water requirements, and 90% construction waste diversion3. With a 200-kilolitre tank that stores rainwater the complex is able to provide for 100% of irrigation demands and 63% of toilet flushing demands3. The indoor air quality draws on outdoor air supply and all air in the Theater building is from a displaced ventilation system3.

Sources:

1 Keniger, Michael et al. “ICC Sydney.” Architecture Australia 106.3 (2017): 50–56. Print.

2 Robert Bird Group. (2021, May 19). International Convention Center (ICC) Sydney. Robert Bird Group. Retrieved February 27, 2022, from https://www.robertbird.com/rbg-projects/international-convention-centre-icc-sydney/

3 ICC Sydney. (n.d.). Environment. ICCSydney. Retrieved February 27, 2022, from https://www.iccsydney.com.au/about/csr/environment

4 Marketing and Design, M. O. O. (n.d.). ICC Sydney. Delta Group. Retrieved February 27, 2022, from https://www.deltagroup.com.au/projects/icc-sydney/