Las Americas - Leon. Mexico

Essay By Matthew Villagomez

Las Americas is an affordable housing complex in Leon, Mexico. The project was designed by SO-IL and developed by Leon Municipal Institute of Living. The project was completed in 20211. Surrounded by cheaply built and nearly identical homes, Las Americas is a project hoping to regenerate and improve low-income living options in Leon1.

The building focuses mostly on community. With no two apartments looking the same, a sense of individuality is created within the complex. The apartments are staggered on each floor to follow a figure 8 plan. Las Americas features 6 floors made up of 60 condo units2. There is no heating and cooling mechanical systems due to the Leon temperate climate. Each unit is cooled through cross ventilation from windows that face the courtyards. The building's square footage is 62,614 square feet and cost 2.4 million dollars2. This equates to $40 per square foot3. To reduce cost, the project was designed so that the construction process was worker-oriented. Since there was construction machinery available, the design was changed from a curved form and facade to a more angular fecade3. This sharper form allowed for the building to be constructed with blocks composed of concrete that could be carried and installed by hand. The construction process did not include HVAC systems and an elevator installment which cut costs as well3. Although the concrete blocks used to construct the building contribute to high carbon costs the building design makes up for this through natural ventilation and its high walkable and bikeable location3. With Mexican culture being very connected to openness through living no two units face each other. The units face the courtyard and have views of the neighborhood allowing for an open and connected feeling throughout the apartment complex.

In the architectural engineering work environment, it can be very easy to get lost in pursuing expensive and massive building projects such as world-class stadiums. While these projects are important it’s fair to say that projects like Las Americas are just as significant if not more. Projects like Las Americas remind engineers who we really work for. Not high-end investors or rich governments but communities and individuals. Las Americas puts the accessibility and sustainability of a building and works around cost and the building process to fulfill those values. Solving climate change and housing inequality sound daunting but Las Americas is a reassuring example of how buildings can solve those important issues through engineering and the construction process.

Sources

1 Englefield , J. (2022, February 8). So-il builds Las Americas affordable housing development in Mexico. Dezeen. Retrieved March 2, 2022, from https://www.dezeen.com/2021/04/15/so-il-builds-las-americas-affordable-housing-development-in-mexico/

2 IL: Las Americas social housing. SO. (n.d.). Retrieved March 2, 2022, from http://so-il.org/projects/las-americas-social-housing

3 AMELAR S. Heart of the Matter. Architectural Record. 2021;209(6):58-61. Accessed March 2, 2022. https://search-ebscohost-com.ezproxy.lib.utexas.edu/login.aspx?direct=true&db=voh&AN=150764559&site=ehost-live